FIMsim

Flood Inundation Model Simulation Tool

User Manual — DEM & LULC Modes

Department of Geography and the Environment

The University of Alabama

Surface Dynamics Modeling Lab

Parvaneh Nikrou

# Overview

FIMsim provides two standalone preprocessing modes for DEM and LULC.

| DEM Mode Download and process Digital Elevation Models for any Area of Interest (AOI). Output: (.tif / .asc / .gpkg) | LULC & Manning Mode Download land use/land cover data and assign Manning’s n roughness coefficients. Output: LULC raster + Manning’s n raster (.tif / .asc / .gpkg) |
| --- | --- |

Both modes support multiple AOIs. Results are saved inside a project folder you define at the start.

# Installation

The installation is straightforward. You only need to install Anaconda first. Then, open Terminal on Mac or Anaconda Prompt on Windows and run these commands one by one:

git clone https://github.com/pnikrou/FIMsim.git

cd FIMsim

conda env create -f environment.yml

conda activate FIMsim

python main.py

The first-time setup usually takes around 10 minutes because it needs to download the required packages. After that, whenever you want to open the app again, you can simply run:

cd FIMsim

conda activate FIMsim

python main.py

# DEM Mode

## DEM Mode downloads and processes Digital Elevation Models for one or more Areas of Interest using USGS 3DEP data, including hydro-conditioned DEMs used for HAND-based flood inundation mapping.

## Step 1 — Create or Open a Project

Every session starts with a project. The project folder stores all downloaded files and settings.

- Enter a Project Name.

- Click Browse to choose where the project folder will be created.

- Click Create Project.

## Step 2 — Define Area(s) of Interest (AOI)

An AOI is a polygon shapefile (.shp) or GeoPackage (.gpkg) that defines the study boundary.

- Click “Browse” and select your AOI file. The AOI file may contain one polygon feature or multiple polygon features.

- After the file is loaded, the table will display all features included in the file. In the “Use” column, check the box for each feature you want to process. To process every feature in the file, click “Select all features”. Otherwise, select features individually.

- If you want to add AOIs from another file, click “Add more AOIs”, then click “Browse” again and select the additional AOI file.

- Once you have selected the desired AOI feature(s), click “Add to confirmed AOIs”. After an AOI is confirmed, a folder with the AOI name is automatically created inside the main project folder. All downloaded and processed data for that AOI will be saved in this folder.

- After confirmation, the selected AOIs will appear in the Confirmed AOIs list. By clicking on each AOI, you can view a map preview of the selected area along with general information about it.

- Click “Next step” at the bottom to continue.

## Step 3 — Configure DEM Settings

For each AOI, choose the DEM source and output format:

| 1 | Source | 3DEP — USGS 3D Elevation Program. HAND DEMs. |
| --- | --- | --- |
| 2 | Cell size | Output raster cell size in meters. Default is 10 m. Match this to your flood model’s required resolution. |
| 3 | Output format | GeoTIFF (.tif), GeoPackage (.gpkg), ASCII Grid (.asc). |

For multiple AOIs, each AOI gets its own settings card. Use Apply current AOI’s settings to all to copy settings across all AOIs at once.

## Step 4 — Run

- Click “Run”.

- Progress is shown in the log panel at the bottom.

- When complete, a results list appears. Click any AOI name to see the DEM preview and elevation statistics.

- The DEM file is saved at: <project>/<AOI name>/dem/dem.tif (or .asc / .gpkg depending on your format choice).

# LULC & Manning’s n Mode

LULC Mode downloads land use / land cover data and derives Manning’s n hydraulic roughness coefficients for each AOI.

## Steps 1–2 — Project and AOI

Follow the same steps as DEM Mode (see above). The project and AOI setup are identical.

## Step 3 — Configure Manning’s n Settings

Choose how Manning’s n is assigned across the AOI:

| Fixed (Uniform) | A single Manning’s n value is applied uniformly across the entire AOI. Use this for simple simulations or when land cover variability is not critical. No LULC download is required. |
| --- | --- |
| Varying (Per LULC class) | LULC data is downloaded and each land cover class is assigned its own Manning’s n value. |

## LULC Data Sources (Varying mode)

- NLCD (USA only, 30 m): NLCD

- National Land Cover Database from USGS. Best for USA-based studies.

- ESRI Sentinel-2 (Global, 10 m): Sentinel-2 Global LULC

- Global land cover from ESRI. Use for study areas outside the USA or when 10 m resolution is needed.

## Editing the Manning’s n Table

In Varying mode, a table lists all LULC classes with default Manning’s n values. You can edit any value directly in the table before running:

- Click any value in the Manning’s n column to edit it.

- Default values are based on published hydraulic literature (Chow, 1959).

- Changes apply only to the current session.

## Step 4 — Run

- Click “Run”.

- Progress is shown in the log panel at the bottom.

- When processing is complete, a results list will appear. Click any AOI name to view previews of the LULC raster and the Manning raster, along with a land cover breakdown table showing each class, its area in km², and its percentage of the total AOI area.

- Output files are saved at: <project>/<AOI name>/lulc.tif (raster) and <AOI name>/lulc.ascii (ASCII grid if requested).